EXPERIMENT -1B

Name:- K.Mugunthan 
Reg. No. :- 192512236

Program Code: 
clc; 
clear; 
close all; 
%% QUESTION B: TE10 VERSUS TM11 MODE 
% WR-90 Rectangular Waveguide 
a = 22.86e-3;    % Width (m) 
b = 10.16e-3;    % Height (m) 
L = 100e-3;     % Length (m) 
f = 10e9;      % Operating frequency (Hz) 
c = 3e8;      % Speed of light (m/s) 
mu0 = 4*pi*1e-7; 
eps0 = 8.854e-12; 
omega = 2*pi*f; 
k0 = 2*pi*f/c; 
%% 1. CUTOFF FREQUENCIES 
% TE10 mode 
fc_TE10 = c/(2*a); 
% TM11 mode 
fc_TM11 = (c/2)*sqrt((1/a)^2 + (1/b)^2); 
fprintf('\n--- CUTOFF FREQUENCIES ---\n'); 
fprintf('TE10 cutoff frequency = %.4f GHz\n',fc_TE10/1e9); 
fprintf('TM11 cutoff frequency = %.4f GHz\n',fc_TM11/1e9); 
%% 2. PROPAGATION CONSTANTS

% TE10 propagation constant 
if f > fc_TE10 
	 beta_TE10 = k0*sqrt(1-(fc_TE10/f)^2); 
else 
	 beta_TE10 = 0; 
end 
% TM11 propagation constant 
if f > fc_TM11 
	 beta_TM11 = k0*sqrt(1-(fc_TM11/f)^2); 
else 
	 beta_TM11 = 0; 
end 
fprintf('\n--- PROPAGATION CONSTANTS ---\n'); 
fprintf('TE10 beta = %.4f rad/m\n',beta_TE10); 
fprintf('TM11 beta = %.4f rad/m (real propagation)\n',beta_TM11); 
%% 3. WAVEGUIDE COORDINATES 
Nx = 100; 
Ny = 80; 
Nz = 100; 
x = linspace(0,a,Nx); 
y = linspace(0,b,Ny); 
z = linspace(0,L,Nz); 
[X,Y] = meshgrid(x,y); 
%% 4. TE10 ELECTRIC FIELD DISTRIBUTION 
Ey_TE10 = sin(pi*X/a); 
figure(1); 
surf(X*1000,Y*1000,Ey_TE10,'EdgeColor','none'); 
colorbar;

xlabel('Width x (mm)'); 
ylabel('Height y (mm)'); 
zlabel('Normalized Ey'); 
title('TE10 Electric Field Distribution'); 
view(45,30); 
grid on; 
%% 5. TE10 MAGNETIC FIELD DISTRIBUTION 
Hx_TE10 = -beta_TE10/(omega*mu0)*sin(pi*X/a); 
Hz_TE10 = -pi/(omega*mu0*a)*cos(pi*X/a); 
H_TE10 = sqrt(Hx_TE10.^2 + Hz_TE10.^2); 
figure(2); 
surf(X*1000,Y*1000,H_TE10,'EdgeColor','none'); 
colorbar; 
xlabel('Width x (mm)'); 
ylabel('Height y (mm)'); 
zlabel('|H| (A/m)'); 
title('TE10 Magnetic Field Distribution'); 
view(45,30); 
grid on; 
%% 6. TM11 ELECTRIC FIELD DISTRIBUTION 
Ez_TM11 = sin(pi*X/a).*sin(pi*Y/b); 
figure(3); 
surf(X*1000,Y*1000,Ez_TM11,'EdgeColor','none'); 
colorbar; 
xlabel('Width x (mm)'); 
ylabel('Height y (mm)'); 
zlabel('Normalized Ez'); 
title('TM11 Longitudinal Electric Field Distribution'); 
view(45,30);

grid on; 
%% 7. TM11 MAGNETIC FIELD DISTRIBUTION 
% Normalized spatial field pattern for TM11 
% Valid as a cross-sectional pattern below cutoff; 
% it is not a propagating-wave solution at 10 GHz.

Hx_TM11 = (pi/(omega*mu0*b))*cos(pi*X/a).*sin(pi*Y/b); 
Hy_TM11 = -(pi/(omega*mu0*a))*sin(pi*X/a).*cos(pi*Y/b); 
H_TM11 = sqrt(Hx_TM11.^2 + Hy_TM11.^2); 
figure(4); 
surf(X*1000,Y*1000,H_TM11,'EdgeColor','none'); 
colorbar; 
xlabel('Width x (mm)'); 
ylabel('Height y (mm)'); 
zlabel('|H| (A/m)'); 
title('TM11 Magnetic Field Pattern'); 
view(45,30); 
grid on; 
%% 8. COMPARISON OF CUTOFF FREQUENCIES 
figure(5); 
bar([fc_TE10 fc_TM11]/1e9); 
set(gca,'XTickLabel',{'TE10','TM11'}); 
ylabel('Cutoff Frequency (GHz)'); 
title('TE10 versus TM11 Cutoff Frequency'); 
grid on; 
%% 9. MODE PROPAGATION DECISION 
fprintf('\n--- MODE COMPARISON ---\n'); 
if f > fc_TE10 
	 fprintf('TE10: PROPAGATES at 10 GHz.\n'); 
else

fprintf('TE10: Below cutoff.\n'); 
end 
if f > fc_TM11 
	 fprintf('TM11: PROPAGATES at 10 GHz.\n'); 
else 
	 fprintf('TM11: DOES NOT PROPAGATE at 10 GHz.\n'); 
end 
fprintf('\nConclusion:\n'); 
fprintf('TE10 cutoff is below 10 GHz.\n'); 
fprintf('TM11 cutoff is above 10 GHz.\n');

OUTPUT