EXPERIMENT 1A

Name:- Mugunthan k 
Reg. No. :- 192412236

Program Code: 
clc; 
clear; 
close all; 
%% WR-90 Rectangular Waveguide Parameters 
a = 22.86e-3;    % Width in meters 
b = 10.16e-3;    % Height in meters 
L = 100e-3;     % Length in meters 
f = 10e9;      % Operating frequency = 10 GHz 
c = 3e8;      % Speed of light (m/s) 
mu0 = 4*pi*1e-7;  % Permeability of free space 
eps0 = 8.854e-12;  % Permittivity of free space 
%% Theoretical Cutoff Frequency for TE10 Mode 
fc = c/(2*a); 
fprintf('THEORETICAL RESULTS\n'); 
fprintf('Cutoff frequency fc = %.4f GHz\n', fc/1e9); 
%% Propagation Constant 
if f > fc 
	 beta = (2*pi*f/c)*sqrt(1-(fc/f)^2); 
	 fprintf('Propagation constant beta = %.4f rad/m\n', beta); 
else 
	 beta = 0; 
	 fprintf('Wave does not propagate at this frequency.\n'); 
end

%% Waveguide Coordinates 
Nx = 100; 
Nz = 100; 
x = linspace(0,a,Nx); 
z = linspace(0,L,Nz); 
[X,Z] = meshgrid(x,z); 
%% TE10 Electric Field Distribution 
% Dominant electric field component is Ey 
Ey = sin(pi*X/a).*exp(-1j*beta*Z); 
figure; 
surf(X*1000,Z*1000,abs(Ey)); 
shading interp; 
colorbar; 
xlabel('Width x (mm)'); 
ylabel('Length z (mm)'); 
zlabel('|Ey| (V/m, normalized)'); 
title('TE10 Mode - Electric Field Distribution'); 
view(45,30); 
%% TE10 Magnetic Field Distribution 
% Hx component for TE10 mode 
omega = 2*pi*f; 
Hx = -(beta/(omega*mu0))*Ey; 
figure; 
surf(X*1000,Z*1000,abs(Hx)); 
shading interp; 
colorbar; 
xlabel('Width x (mm)'); 
ylabel('Length z (mm)'); 
zlabel('|Hx| (A/m, normalized)');

title('TE10 Mode - Magnetic Field Distribution'); 
view(45,30); 
%% Electric Field Cross-Section 
figure; 
plot(x*1000, sin(pi*x/a),'LineWidth',2); 
grid on; 
xlabel('Width x (mm)'); 
ylabel('Normalized Electric Field'); 
title('TE10 Electric Field Across Waveguide Width'); 
%% Summary 
fprintf('\nSIMULATION SUMMARY\n'); 
fprintf('Operating frequency = %.2f GHz\n',f/1e9); 
fprintf('Cutoff frequency = %.4f GHz\n',fc/1e9); 
fprintf('Propagation constant beta = %.4f rad/m\n',beta); 
fprintf('TE10 mode propagates because f > fc.\n');

OUTPUT